UBUNTU MUNICIPALITY

TECHNICAL REPORT

EXTERNAL STORM WATER

MASTERPLAN:

VICTORIA WEST, RICHMOND &

LOXTON

Prepared for:	Prepared by:

JULY 2009

Ubuntu Municipality	Stabilis Development (Pty) Ltd

Private Bag x 329	P.O. Box 861

Victoria-West	Kimberley

7070	8300

Tel no: 053- 621 0026	Tel no 053-8331654

Fax no: 053-621 036$	fax no 053-8313786

E-mail info@stabilis.co.za

1

CONTENTS	PAGE

- Background	2

- Introduction

2.1 Purpose of Technical report	2

2.2 Project objectives	2

2.3 Project Outcomes	3

- Proposed Project

3.1 Project name	3

3.2 Location	3

3.3 GIS Reference	3

3.4 Purpose of Project Technical Report	4

4	Project Beneficiaries

4.1 Background	4

4.2 Beneficiaries	4

5.	Scope of the works

5.1 Brief description of Project	4

5.2 Climatic and Hydrological Conditions	5

5.3 Design Parameters	5

5.4 Implementation	5

5.5 Implementing Agent	5

5.6 Funding plan	6

5.7 Budget proposal	6

5.7 Estimated Costs	6

5.8 Maintenance	6

ANNEXURE A	Figure SK198511 Locality Plan

Figure SK 1985/1 Locality Plan Victoria West	7

Figure SK 1985/2 Locality Plan Richmond	7

Figure SK 1985/3 Locality Plan Loxton	7

2

- BACKGROUND

The towns Richmond, Victoria West and Loxton are located within the jurisdictional area of Ubuntu. municipality — see Annexure A: Figure SK1985/1. Richmomd and Loxton are situated approximately 112 and 81 kilometers from Victoria-west, the main centre of Ubuntu municipality, respectively.

The towns, or par of the towns, are often flooded by storm water run-off caused by typical thunderstorms in this area and due to a lack of proper external storm water drainage facilities. The lack of external storm water facilities has a negative impact on the building of new houses and service delivery in general.

The bridge in Victoria-West that links the business centre with the residential area in Victoria-West was damaged by flood water. Some of the houses in Richmond were flooded in 2008 due to a lack of proper external drainage facilities. The flooding of residential properties and the damages to other infrastructure, especially the existing gravel roads and streets, is a concern for the municipality and a nuisance to the residents.

It is the intention of Ubuntu municipality to apply for funding from the MIG program to design and construct proper external storm water facilities to manage the run-off from rain storms. The objectives of the Storm Water master Plan are to identify projects proposals for each of the towns for implememtation.

- INTRODUCTiON

Ubuntu municipality invited suitable service providers, by an open procurement process, to submit proposals for compiling a storm water master plan for the three towns. The storm water master plan will enable Ubuntu Municipality to:

- Identify the storm water problem areas in each town

- Compile project proposals for the identified problem areas

- Prioritize project proposals

- Compile estimated cost estimates for each prioritized project proposal

- Prepare technical proposals for each prioritized project for funding on the MIG or similar funding program for implementation

2.1 PURPOSE OF TECFINICAL REPORT

This purpose of this technical report is to briefly describe the scope of the proposed project, the proposed methodology to be followed, process for implementation and the estimated project costs.

2.2 PROJECT OBJECTIVES

The main objectives of this proposed project are the following:

- To compile a storm water master plan for Victoria West, Richmond and Loxton. The master plan will be used to prioritize project proposals for solving the external storm water problems in each of the towns.

3

- .The master plan will also indicate areas that are unsuitable for future
development, especially for residential and similar extensions to the towns.

- To apply for funding to implement the prioritized external storm water projects in the three town

- Involve the relevant Community in the project. The community will contribute to the research work by their inputs during the field work.

2.3 PROJECT OUTCOMES

The expected outcomes for the project are the following:

- Storm Water Master Plan compiled for Victoria-West, Richmond and Loxton.

- Project proposals and cost estimates for towns completed

- Prioritized project proposals approved by Ubuntu municipality and included in their IDP review

- Involvement of local community in the project process. 3. PROPOSED PROJECT DESCRIPTION

3.1 PROJECT NAME	STORM WATER MASTER PAN FOR

VICTORIA-WEST, ROCIIMOND AND LOXTON.

The proposed project mainly entails the compiling of a master plan for external storm drainage facilities for the three towns.

3.2 LOCATION

The location of the proposed project is in the Ubuntu Municipal area at the towns of Victoria-West, Richmond and Loxton.

| 3.3 GIS REFERENCE |  |  |  |
| --- | --- | --- | --- |
| District: | Pixley Ka-Seme |  |  |
| Nearest town: | De Aar |  |  |
|  | Victoria-West | Richmond | Loxton |
| Longitude: | 31' 24" 16,6' | 31' 24" 49,5' | 31' 28" 40,5' |
| Latitude: | 23' 07" 01,8' | 23' 56" 37,4' | 22' 21" 11,6' |

4

3.4 PURPOSE OF PROJECT BUSINESS PLAN

The main purpose of this technical report is to support the MIG application for funding of this proposed project and to briefly describe the proposed project and the processes to be followed for implementation.

4	PROJECT BENEFICIARIES

4.1 BACKGROUND

The towns of Victoria-West, Richmond and Loxton are often flooded and properties, roads and other infrasttucture damaged by storm water run-off due the lack of proper external storm water drainage facilities. The storm water master plan,

specifically for the external facilities, will also be used to assist in the planning of future town planning and extensions to the existing towns.

4.2 BENEFICIARIES

The beneficiaries of this project are mainly:

- Residents of Victoria-West, Richmond and Loxton.

- . Service delivery will be maintained at an acceptable level due to the management of storm water run-off.

- The unemployed people from these areas will benefit from the creation of jobs and the transfer of skills during the implementation phases of this project.

5 SCOPE OF THE WORKS

The proposed project will include the following project elements to ensure that the required outcomes will be met:

5.1 BRIEF DESCRIPTION OF PROJECT.

The proposed project comprise of the following main elements:

- Contour survey of each town as required to execute the project

- Evaluation of existing external storm water facilities

- Identify preliminary project proposals for the external storm water management.

- Preliminary planning and design of identified project proposals by modeling the storm water run-off

- Preliminary cost estimates of identified project proposals and the prioritization of the project proposals

- Technical report containing the Storm Water Master Plan with prioritized projects and estimated costs per project proposal.

5

5.2 CLIMATIC AND HYDROLOGOCAL CONDITIONS

The Ubuntu area is located in the summer rainfall region of South Africa with typical thunder storms. The average annual rainfall (Victoria-West) is 253 mm with a 24-hour maximum recorded rainfall of 80mm. Approximately 80 % of the average annual rainfall occurs during the months of October to April.

The catchment characteristics vary from hard and rocky surface with a low permeability to a sandy soil with a medium permeability. The vegetation is very scarce and entails natural grass, typical Karroo bush and scrubs in the undeveloped areas with no vegetation in the developed areas.

These catchment characteristics, together with high intensity rainfall, are the

main cause of the high run-off peaks although this area has a relative low average annual rainfall.

The natural gradient of the area vary from steep on the mountainous areas while the areas in the town is mainly flat.

5.3 DESIGN PARAMETERS

The preliminary design and planning of the external storm water facilities will be done according to "design Flood Determination in South Africa", issued by the HRU, Wits University and the guidelines contained in the "Human Settlement Planning and Design" compiled by the CSIR.

5.4 IMPLEMENTATION

The implementation of the project will include the following important phases:

- Contour survey of towns and surrounding areas to compile contour maps for each town that can be utilized for storm water modeling, preliminary planning and design purposes.

- Identification of problem areas in each town, project proposals and cost estimates per project proposal.

- Prioritizing of project proposals for each town

- Compile Storm Water Master Plan report for approval by Ubuntu Municipality

5.5. IMPLEMENTATION AGENCY

Ubuntu Municipality will be the Implementing Agency and the responsible agent.

- Contact person:	Mr ME Finis
Municipal Manager

6

- Address:	Ubuntu Municipality
Private Bag X 329 Victoria-West

7070

- Telephone number:	053-621 0026

- Fax number	053-621 0368

5.6 FUNDING PLAN

All funds must be made available to the Municipal Manager, Ubuntu Municipality who will, in accordance with Standard Procedures, take responsibility for the administration of all funds. Application for funding of this project will be submitted to the MIG office, Department of Local Government and Housing, Northern Cape.

5.7 BUDGET PROPOSAL

The preliminary budget for the proposed project in the municipal area is as follows:

5.8 ESTIMATED PROJECT COST:

The estimated project cost is the following and based on the tender accepted:

- The contract amount does not include any escalation clause

|  | Contour surveys & compile contour maps Engineering fees: Evaluation of existing storm | R 90 400-00 |  |
| --- | --- | --- | --- |
|  | water infrastructure | R | 6 620-00 |
|  | Preliminary planning, design & modeling | R | 21 000-00 |
|  | Compile report: Storm Water Master Plan | R | 21 000-00 |
|  | Submit final report | R | 9 000-00 |
| 0 | Expenses on travelling, etc | R | 45 360-00 |
|  | ESTIMATED PROJECT AMOUNT | R 193 380-00 |  |

14 % VAT	R 27073-20

TOTAL ESTIMATED PROJECT AMOUN R220 453-20

5.9 OPERATIONS AND MAINTENANCE

The operations and maintenance of the project after completion is the responsibility of Ubuntu Municipality. This project, the storm water master plan, will have no operational or maintenance requirements.

5.10 TRAINING

No training will be applicable to this project, the storm water master plan, as training will only be required at the implementation stage of the identified and prioritized MIG projects.

7

Accredited training will then be provided as per the slas audit conducted by Ubuntu Municipality in collaboration with the appointed service providers.

For: MUNICIPAL MANAGER:	CHRIS BOTHA Pr Eng

UBUNTU MUNICIPALITY	STABILIS DEVELOPMENT (PTY) LTD

| ANNEXURE A SK 1985/1 - LOCALITY PLAN |  |  |
| --- | --- | --- |
| SK 1985/2 - | Locality Plan | Victoria-West |
| SIC 1985/3- | Locality Plan | Richmond |
| 8K 1985/4- | Locality Plan | Loxton |