Privaatsak X 329, VICTORIA WES, 7070, Tel: 053 6210026, Fstks: 053 6210368

TECHNICAL REPORT

| MUNISIPALITEIT MUNICIPALITY U-MAS IPALA |  |
| --- | --- |

UPGRADING OF STREETS IN VICTORIA WEST,

LOXTON & RICHMOND:

PHASE A

Prepared for:	Prepared by:

DATE: 04t NOVEMBER 2010

| Ubuntu Municipality Private Bag X329 VICTORIA WEST 7070 | Era Engineers (Pty) Ltd PO Box 4344 DURBANVILLE 7551 |  |
| --- | --- | --- |

Tel: 053 621 0026	Tel: 021 975 6200

| Ubuntu Municipality; ERO. Upgrading of Streets in Victoria West, Richmond & Loxton | Technical Report |
| --- | --- |
| Table of Contents Item Description |  | Page |
| 1.1 | GENERAL | 'I |
| 1.2 | TERMS OF REFERENCE | 1 |
| 2.1 | PROJECT STEERING COMMITTEE (PSC) | 2 |
| 2.2 | IMPLEMENTING AGENT AND PROJECT MANAGER | 2 |
| 2.2.1 | Implementing Agent | 2 |
| 2.2.2 | Project Manager and Design Engineer | 2 |
| . . 3.1 | BRIEF PROJECT DESCRIPTION | 3 |
| (•_-) 3.2 | PROJECT LOCATION | 3 |
| 3.3 | DESCRIPTION OF STREETS UNDER INVESTIGATION | 3 |
| 3.4 | CLIMATIC CONDITIONS | 4 |
| 4.1 | TRAFFIC SURVEYS | 4 |
| 5.1 | GEOMETRIC DESIGN | 4 |
| 5.1.1 | Typical Road Cross-section | 4 |
| 5.12 | Horizontal Alignment | 5 |
| 5.1.3 | Vertical Alignment | 5 |
| 5.1.4 | Intersections | 5 |
| 5.1.5 | Road Safety and Road Signs | 5 |
| r 5.1.6 | Services | 5 |
| --- 6.1 | BACKGROUND | 6 |
| 6.2 | ASSESSMENTS AND SURVEYS | 6 |
| 6.2.1 | Visual Assessment | 6 |
| 6.3 | MATERIALS INVESTIGATION | 6 |
| 6.4 | STRUCTURES | 6 |
| 6.4.1 | Drainage Structures | 6 |
| 7.1 | SCOPE | 7 |
| 7.2 | NEW PAVEMENTS | 7 |
| 7.3 | ROAD FURNITURE, ROAD SIGNS AND ROAD MARKINGS | 7 |
| 8.1 | ACCOMMODATION OF TRAFFIC | 8 |
| 82 | PROVISION OF EDGE BEAMS | 8 |

| Ubuntu Municipality Upgrading of Streets in Victoria West, Richmond & Loxton |  | Technical Report |
| --- | --- | --- |
| Item | Description | Page |
| 8.3 | CONSTRUCTION MATERIALS | 8 |
| 8.4 | CONSTRUCTION MONITORING | 8 |
| 8.5 | EXISTING SERVICES . | 8 |
| 9.1 . | CONSTRUCTION COST ESTIMATES | 9 |
| 9.2 | FUNDING SOURCES | 9 |
| 9.3 | OPERATION AND MAINTENANCE COSTS | 9 |
| 9.4 | PROVISIONS FOR OPERATING AND MAINTENANCE COSTS | 9 |
| 9.5 | PROPOSED JOB CREATION AND TARGETED PROCUREMENT | 10 |
| 9.5.1 | Local Labour | 10 |
| ( i . |  |  |
| \- ' 9.5.2 | Targeted Procurement | 10 |

APPENDICES

Appendix A: Locality Plan

Appendix B : Appointment Letter

Appendix C: Cross Sections and Layout Drawings Appendix D : Breakdown of Construction Costs

|  | ERO 1 ENGENt*.1, ii |
| --- | --- |

Ubuntu Municipality

Upgrading of Streets in Victoria West, Richmond & Loxton	Technical Report

1.1	GENERAL

This Technical Report in respect of the proposed upgrading of streets in Victoria West,
Richmond and Loxton is submitted by virtue of the authority conferred by the Ubuntu
Municipality.	This is done in accordance with the appointment letter Reference No.

dated 20 October 2010.

In accordance with the ToR, the main objective of the investigation is to determine the most appropriate strategy for the upgrading of the streets in the area under investigation. This report covers the general aspects of the investigation and provides inter alia detail on the existing pavement condition, construction works and cost. The main findings and recommendations are summarised in this report. A project locality plan is presented in Appendix A.

1.2 TERMS OF REFERENCE

The terms of reference the appointment of Ero Engineers (Pty) Ltd [ERO] include inter alia the following:

Planning for upgrading of selected gravel roads in Victoria West, Richmond and Loxton.

Liaison and consultation with communities and various stakeholders in the affected area.

- Preparation and submission of a report embodying preliminary proposals, estimates of cost

and time.

- Compilation of specifications for the construction of the streets identified by the Ubuntu Municipality for implementation.

Basic Planning and Detailed Design of the Project.

Preparation of all Documents necessary to enable Tenders for the Works to be called for. Preparation of any further Plans, Designs and Drawings which may be necessary for the execution of the works.

Advertising of tenders and evaluation thereof.

Contract Administration and Construction Supervision.

Compilation of As-built Drawings and Completion Reports.

Conduct an Environmental Impact Assessment if necessary.

The Appointment Letter is presented in Appendix B.

|  | (110 ENGI ERS |
| --- | --- |

Ubuntu Municipality

Upgrading of Streets in Victoria West, Richmond & Loxton	Technical Report

2.1 PROJECT STEERING COMMITTEE (PSC)

The PSC will represent all the representatives in the community of Victoria West, Richmond and Loxton. Through the PSC, every community member may therefore make an input in the management of the project. Ubuntu Municipality, in its capacity as the implementing agent, is the principal stakeholder and is also represented on the PSC. The project managers, ERO, are also represented on the PSC. However, their role is to provide technical and managerial advice to the members of the PSG.

2.2 IMPLEMENTING AGENT AND PROJECT MANAGER 2.2.1 Implementing agent : Ubuntu Municipality

Contact person :	Mr M Fillis (Municipal Manager)

Address	Ubuntu Municipality

Private Bag X329 VICTORIA WEST 7070

Telephone no.	(053) 621 0026

Facsimile no.	(053) 621 0368

Email	mfillisamweb.caza

2.2.2 Project Manager and Design Engineer : Ero Engineers

Contact Person :	Mr Kobus Visser

Address	Ero Engineers (Pty) Ltd

PO Box 4344

DURBANVILLE

7551

Telephone no.	•

(021) 975 6200

Facsimile no.

(021) 975 6400

Email	info@ero-engineers.co.za

|  | ERO 2 |
| --- | --- |

| Ubuntu Municipality | E RO IENGi 1:4 (F.,..114 | 3 |
| --- | --- | --- |

Upgrading of Streets in Victoria West, Richmond & Loxton	Tech nicai Report

Other key personnel in the proposed project team are as follows:

Mr 0 Jonker	Documentation, Pavement Design

Mr L Kriegler	Geometric Design, Stormwater Design

Mr F Henning	Construction Monitoring

3.1 BRIEF PROJECT DESCRIPTION

ERO's appointment called for the Upgrading of unpaved streets within Victoria West, Richmond and Loxton. The proposed construction work will inter aria include the following actions:

TABLE 2.1 : SCOPE OF WORKS

| Construction Activity | Description |
| --- | --- |
| Local residential streets | - Importing material from new borrow pit. - Construction of stabilised C4 subbase and imported selected layer. - Surfacing —80 mm interlocking pavers. |
| Drainage Improvements | - Concrete V-drains, kerbs, etc. |
| Road Furniture | - Road signage as required. |
| Road Markings | - Road markings on completion of road works. |

3.2 PROJECT LOCATION

This project lies in Northern Cape Province and includes the towns of Victoria West, Richmond and Loxton.

3.3 DESCRIPTION OF STREETS UNDER INVESTIGATION

The streets under investigation are located within the developed areas. There are major stormwater drainage problems related to these streets as they are not paved and slopes are very steep. Another problem is the fact that these roads were never actually constructed as

Ubuntu Municipality

Upgrading of Streets in Victoria West, Richmond & Loxton	Technical Report

roads, rather areas that were cleared and used as roads. Refer to layout plans in Appendix C indicating the streets of concern. No signalised intersection occurs.

3.4 CLIMATIC CONDITIONS

Rainfall Station Longitude

Height

Average Rainfall Average No of Days with rainfall exceeding 10 mm

Beaufort West (00920815) 32°21'23" ; Latitude : 22°34'53" 840 mm above sea level

262 mm per year

6 days / years

t

3.5 TOPOGRAPHY

The towns lie in the typical Karoo region with a flat to undulating topography. 4.1 TRAFFIC SURVEYS

No detailed traffic counts are currently available. The cumulative equivalent design traffic loading will therefore be done by means of estimation from tabulated traffic classes (Draft UTG3 Structural Design of Urban Roads — 1988).

5.1 GEOMETRIC DESIGN

The geometric design of the streets will be done in accordance with the recommended qesign standards. It is therefore envisaged to apply geometric improvements with the intention being to improve the safety of all road users.

5.1.1 Typical Road Cross-section

The proposed new streets consist of single carriageway surfaced streets. The typical cross section varies according to the classification. Detailed cross sections are presented in Appendix C.

|  | RO ENG! NF f„115 | 4 |
| --- | --- | --- |

Ubuntu Municipality

Upgrading of Streets in Victoria West, Richmond & Loxton	Technical Report

Surface drainage will be accommodated on the streets at selected positions. Kerbing will consist of a combination of barrier kerbs and edge beams along the road. Stormwater runoff will be discharged on the surface and no kerb inlets or stormwater pipes will be provided. A lined trapezoidal drain is proposed on one side of the road.

5.1.2 Horizontal Alignment

The local residential streets will be designed for a speed of 50 km/h. The super-elevations on all the horizontal curves will be kept to a two percent reverse camber as far as possible, as a direct result of the urban setting of the road and the low speeds. It is not foreseen that any future improvement to the horizontal alignment would be required.

-} 5.1.3 Vertical Alignment

The streets are situated in a residential area, and the existing vertical alignment will be maintained as far as possible.

The minimum K-values and other vertical design criteria will be according to the normal standards. The presence of existing services in the road reserve will have to be addressed by the vertical alignment.

5.1.4 Intersections

(77	There are no major intersections within the study area.

5.1.5 Road safety and Road Signs

Road signage and road markings will be provided where applicable to improve safety. 5.1.6 Services

All visible services along the entire area must be noted during the visual site investigation. Should major relocation of existing services occur, the horizontal alignment of the road will not deviate from the existing position.

A detailed survey of the existing services will be done in the detail design stage of this project.

|  | E RO ENGINEERS 5 |
| --- | --- |

Ubuntu Municipality

Upgrading of Streets in Victoria West, Richmond & Loxton	Technical Report

6.1 BACKGROUND

A preliminary visual inspection was done during September 2010. A detailed visual inspection will be done during the final design stage.

6.2 ASSESSMENTS AND SURVEYS 6.2.1 Visual Assessment

Pavement

The objective of the detailed visual inspection on the proposed streets is to determine the condition of the existing roads and associated problem areas.

Drainage

Stormwater accommodation, drainage of water on the road surface as well as drainage next to the streets will be inspected.

No ponding of water should be allowed next to the road and should be diverted away from the pavement layers.

6.3 MATERIALS INVESTIGATION

A reputable materials laboratory will be contracted to do a detailed materials investigation on the area under investigation as well as the borrow areas.

• 6.4 STRUCTURES 6.4.1 Drainage Structures

Drainage along the streets will be managed by means of kerbing and concrete trapezoidal drains.

|  | E RO lENIN,1„!„1 | 6 |
| --- | --- | --- |

Ubuntu Municipality

ERG

;ENCINEE11.5

7

Upgrading of Streets in Victoria West, Richmond & Loxton	Technical Report

7.1 SCOPE

The design approach for this project will be based on the following principles:-

To optimise the use of in-situ materials.

To provide an appropriate pavement structure for the selected design life.

The design must be economical and cost effective also in terms of maintenance operations.

7.2 NEW PAVEMENTS

7.2.1 The following pavement structure is considered:

- Utilise of the in-situ pavement material for selected layers and roadbed.

- Import subbase material from borrow pit and surface with 80 mm interlocking pavers.

In keeping with the above, the following preliminary designs are proposed:

| Surfacing ,., | 80 mm Interlocking Pavers (25 MPa) |
| --- | --- |
| Subbase . | 150 mm Stabilised material (C4) compacted to 97% of mod. AASHTO density. |
| Selected layer | 125 mm Natural gravel (G5) compacted to 95% of mod. AASHTO density. |
| Roadbed | 125 mm Natural gravel (G8) compacted to 93% of mod. AASHTO density. |

7.3 ROAD FURNITURE, ROAD SIGNS AND ROAD MARKINGS

The newly constructed streets need to be furnished with road signs. Road markings along the entire alignment will be constructed. No allowance will be made for the provision of electrical lights adjacent to the streets.

Ubuntu Municipality

Upgrading of Streets in Victoria West, Richmond & Loxton	Technical Report

8.1 ACCOMMODATION OF TRAFFIC

The construction of the pavement layers and shoulders will be done in half widths where possible to accommodate traffic (local residents with properties adjacent to the road).

The speed of vehicles alongside actual construction areas must be limited to a maximum of 20 km/h. STOP/GO signs and flagmen shall be employed at the points where construction traffic enters the working area thus creating a potential danger situation for public traffic.

8.2 PROVISION OF EDGE BEAMS

Numerous accesses to properties adjacent to the streets have been observed. Provision has been made in the cost estimate to provide for edge beams and kerbing.

8.3 CONSTRUCTION MATERIALS

The use of in-situ materials will be maximised for the construction of the pavement layers. 8.4 CONSTRUCTION MONITORING

The extent of the work will not warrant full time site staff for construction monitoring. Although it will be finally agreed between the officials of the Ubuntu Municipality and the consulting engineer before commencement of construction, it is recommended that construction monitoring be done at Level 2. This level is in accordance with the duties of the consulting engineer pertaining to construction monitoring as defined in Government Gazette No 31749, 02 January 2009.

8.5 EXISTING SERVICES

Relocation/protection of existing services is anticipated on the project. Provision for this has been made in the cost estimate.

Application for exemption from a full EIA will be made due to the fact that the streets run along the existing disturbed area.

|  | E RC) ENGINEE,RS 8 |
| --- | --- |

Ubuntu Municipality

ERO,ENGINE.U.5	9

Upgrading of Streets in Victoria West, Richmond & Loxton	Technical Report

9.1 CONSTRUCTION COST ESTIMATES

It

The total estimated cost to upgrade the local residential streets is detailed below:

| Description | Total km | Amount |
| --- | --- | --- |
| Major Collector Street | 8.27 | R 36 801 500.00 |
| Access Loop | 2.25 | R 8 691 750.00 |
| Access Street | 0.83 | R 3 087 600.00 |
| Sidewalk brick paved | 8.27 | R 4 217 700.00 |
| Sidewalk sealed | 2.25 | . R 607 500.00 |
| Sidewalk gravel | 0.83 | R 186 750.00 |
| Total cost roadworks |  | R53 592 800.00 |
| Training |  | R200 000.00 |
| Fees (approx 12,5%) |  | R6 750 000.00 |
| Total cost |  | R60 542 800.00 |
| VAT |  | R8 475 992.00 |
| Total estimated cost |  | R69 018 792.00 |

9.2 FUNDING SOURCES

Municipal Infrastructure Grant (MIG)

r- 9.3 OPERATION AND MAINTENANCE COSTS

No major operational and maintenance costs will be applicable on this road network within the first four years after construction; however should minor defects occur within this time, the Ubuntu Municipality will be responsible to take corrective action.

9.4 PROVISIONS FOR OPERATING AND MAINTENANCE COSTS

9.4.1 The Engineering Department of Ubuntu Municipality will be responsible for all operating and maintenance aspects.

Ubuntu Municipality

Upgrading of Streets in Victoria West, Richmond & Loxton	Technical Report

9.4.2 The estimated annual operating and maintenance costs will be included in the tariff structure implemented by the Local Authority.

9.5 PROPOSED JOB CREATION AND TARGETED PROCUREMENT 9.5.1 Local Labour

A condition of tender will be that the Contractor must make use of labour intensive construction methods as far as possible.

It will also be stated in the contract that Contractors shall utilise local labour as much as practically possible and the percentage thereof will be determined by the Ubuntu Municipality in conjunction with the consulting engineers.

9.5.2 Targeted Procurement

The procurement of contracting entities will be done using the targeted procurement procedures. The tender document will be compiled in accordance with Target Procurement Specifications. The tenders will be drawn up in such a way as to ensure access for sub-contactors and local labour.

|  |  |  | 01 —1 \ |
| --- | --- | --- | --- |

FJJ1VSER	DATE

E RO -NGIN

10

Ubuntu Municipality

Upgrading of Streets in Victoria West, Richmond & Loxton	Technical Report

APPENDIX A:

RO	N,tu

Locality Plan

C •

arkas

Sada .

Baboon

S.FIelen

Paternostor,.!:,

VredenhurX'

Saldanha

SpirierM.a 5-ay

tangeba

wesa klaRi re flesen.

eacon Bay

ST LONDON

ma Point

Daseen Wan

Hota

,

Vander	L-

Vioolsdrif

a rrento	,

7 FreeStat	tel

/ Melia?	0,UM

,	d

BERLE!? oodder

MON,

Port Nolloth	Steinko

- Wedge Point\	111j.i.kab

N o. r;lit h

EiEsats3f,;"

ok

qt11-4877.5.

toe

ogserov''.1".4

I	"

1/0

4w..5;:b—L	Eng&

* • eenK 17W.iEshowec; '

"St	t	TillskoP

0	„

o 0

ta

ongat

- wn

DURBA

sipifigo
*Amanzimtot

Umlcomaas

Sottrourgh

Sezela

thberdene

)edk: COI	WIll Pqrs

Port Shopstone

Ramag vonseo

art Edward

kamnAi :mire qesor.)e

1

Kalkio ind

akho.

Faureamith

Vanderea.

- •

- Gvir•

3,ir4fopteinpunt

Hondedipbaai0, „

Sfrandionteinpunt \

Island Point'L\

delbur

1IlOrt SLIJohns

Slik4 Masitre Reserve

1-041eira, Natnra Reserve

Rooduin

Lambettl; Say

Melkb astrend	Cof„,,6t,Mon

'	at§

itoe;loot.rir

M

4,4r-

.	ftrr4,4

'

CAPE T	•

Hou BayC	TAPIIIIF,4,4ga7P-T,P

P

-1eXte1tbary.

180401134..Krssrqi

eVece(Xer.umpum)

•

1

rnanado CoprSt Francis

acde

11.4.	.49111.-	c.,n°

Simon's' own 4.,..,;.S.tra,,

dD4 1'MM:1.4 Nal:c%,.01FwF

Cape tFpdHope

GanS

•

E RO I ENGINEERS

(Pry) Ltd

Tat + 27 21 975 6200 Fax + 2721 975 6400
E-mail lnfogero-enginears.co.za

Oxford Gate, Block B,54 Oxford Street, Durbanville, 7550,
P.O. Box 4344, DurbanvIlle, 7551

UPGRADING OF STREETS: UBUNTU MUNICIPALITY

File name

N.T.S.

LOCALITY PLAN

Drw No.

ER 1006/01

Ubuntu Municipality

Upgrading of Streets in Victoria West, Richmond & Loxton	Technical Report

APPENDIX B:

Appointment Letter

'ty

| MUNISIPALITEIT MUNICIPALITY U-MASIPALA |  |
| --- | --- |

UIPUNT11--

Privaatsak X 329, VICTORIA WES, 7070, Tel: 053 6210026, Faks: 053 6210368

| VISION The provision of efficient, sustainable and affordable services to the inhabitants of IThuntu within a safe environment |  |  |
| --- | --- | --- |
| U VERWYSING: YOUR REFERENCE: ER1006 |  | ONS VERWYSING: OUR REFERENCE: MF FILLIS |

E-MAIL ADRES: ubuntuvic©wsinetco.za 20 October 2010

ERO Engineers (Pty) Ltd P.O. Box 4344

Durbanville

7551

ATTENTION Mr FJJ Visser

LETTER OF APPOINTMENT: CONSULTING ENGINEER: UPGRADING OF STREETS IN VICTORIA WEST, RICHMOND AND LOXTON

We hereby confirm that ERO Engineers (Pty) Ltd has been appointed to perform the civil engineering services of the abovementioned project.

The appointment is based on the following attached agreement:

- CESA Standard Form of Agreement for Consulting Services.

- ECSA Guideline Scope of Services and Tariff of Fees.

The scope of works includes provision professional civil engineering services in connection with the upgrading of unsurfaced streets in the towns of Victoria West, Richmond and Loxton under the authority of Ubuntu Municipality..

The council resolution number is	 and the IDP number is

We trust you will find the above in order.

Yours faithfully.

MF FILLIS Municipal Manager

Ubuntu Municipality

474.IN

Upgrading of Streets in Victoria West, Richmond & Loxton	Technical Report

APPENDIX C:

Cross Sections and

Layout Drawings